GEOLOGICAL SURVEY OF CANADA

OPEN FILE 3882

### Geochemical Composition of Hummock and Hollow Peat

and Feather Moss in the Vicinity of the Horne Smelter,

### Rouyn-Noranda, Quebec

#### INTRODUCT ION

In 1997 Geological Survey of Canada initiated research on heavy metal concentrations in peatlands in the vicinity of the Horne smelter at Rouyn-Noranda, Quebec.  This work was carried out as part of the “Point Sources” Sub-programme in the Metals of the Environment (MITE) programme.    The purpose of the Point Sources initiative was to determine and compare the physical and geochemical record of particulate smelter emissions in surficial media (till, soils, lake sediments, snow, lake waters, peat, and tree rings) and to evaluate the impact of anthropogenic loading on these natural systems.

The peatland project in Rouyn-Noranda was undertaken in two phases.  The first phase, started in 1997, consisted of a sampling survey of 41 peatlands located within a 100-km radius of the smelter.  At each peatland site, hummock peat samples were collected and, where possible, cores were collected through peat to mineral sediment in the adjacent flat part of the peatland, also called the hollow.  Peat samples from 24 of the 41 sites were analyzed for selected trace, minor, and major elements, their content of organic matter and bulk density. Also at 15 of the 24 sites, surface feather moss (Pleurozium schreberi), a useful bioindicator of heavy metals, was collected and analyzed for the same suite of elements. The second phase, ongoing since 1999, has involved detailed work on the distribution and character of metals in hummock peat from four sites located 10,12, 38 and 130 km away from the smelter.  In this report, geochemical and other data for peat, underlying mineral sediments, and feather moss from the 24 “first phase” peatlands are reported.

# METHODOLOGY

Site selection - Sample sites were selected, based on peatland distribution, dominant wind directions, and road access.  In the 1997 survey, samples were collected at 41 sites (97KFA3500 to 97KFA3540) but samples from only 24 of the 41 sites were analyzed (Fig. 1).  The data for samples from these 24 sites form the basis of this report.  Sites ranged from 7 to 92 km from the smelter (Appendix B).  At each site, the peatland setting, vegetation, and location were described (Appendix B).  The majority of sites were ombrotrophic (bog) or weakly minerotrophic peatlands (poor fens) (Zoltai, 1988b).  The surface vegetation was predominantly Sphagnum spp. And ericaceous shrubs (Kalmia angustifolia (sheep laurel), K. polifolia (bog laurel), Ledum groenlandicum (Labrador tea), Andromeda glaucophylla  (glaucous bog rosemary), Chamaedaphne calyculata (leather leaf).

Hummock sampling - At each site, the tallest available Sphagnum hummock covered with the fewest number of shrubby plants was selected for sampling.  From top to bottom in the hummock, samples were collected at about 2 to 4 cm intervals with stainless steel clippers, and, where possible, the volume of each sample was estimated.  Peat characteristics of the hummock sequence were described in the field (Appendix C).  Measurements of pH were taken at or near the hummock base.  In the data lists, the hummock sequences are identified with the peatland site number plus an “h” (e.g. 97KFA3500h).

Hollow sampling - At sites where there were peat accumulations of 30 cm or more in thickness in the flat part of the peatland (hollow) adjacent to the hummock, a second set of samples was collected.   From the surface to a depth of approximately 30 cm, block samples were hand-cut and, where possible, sample volumes were estimated.  Below 30 cm, peat was collected to mineral sediment using a 4.5 cm diameter stainless steel macaulay corer. The hollow sequences were described in the field (Appendix C).  At the time of collection the hollow core was sub-sectioned into 10 to 50 cm long segments for analysis.  Mineral sediments from the base of the core were also sampled.  The hollow sequences are identified with the peatland site number plus an “f” (e.g. 97KFA3500f).

Feather moss sampling - At 14 sites, feather moss (Pleurozium schreberi) (Fig. 5) was collected from the hollow, as close to the sample hummock as possible.  Using methodology described by Rasumussen (1994), 2 to 4 cm long segments were clipped from the ends of the moss branches with stainless steel clippers.

Laboratory Processing - Peat hummock and hollow samples were assigned a laboratory number, coded using a Munsell soil colour chart (Munsell Color Company, 1971), weighed, and put in the oven to dry at 37degrees C.  The dried samples were then reweighed and the moisture contents, based on both wet and dry weights, and bulk densities calculated (Appendix D).

From the poorly decomposed surface hummock samples, two and, where there was sufficient material, three splits (> 1gm) of the original dried sample were extracted.  From one of these splits, Sphagnum fragments were hand-picked and the resulting sub-sample was identified with the laboratory number plus “s”. The remaining one or two splits were both identified with the laboratory number plus “w”. From the samples of more decomposed  peat collected at greater depths in the peat hummock, two splits (>1 gm) of the original dried sample were extracted and both identified with the laboratory number plus “w”.  From the hollow samples, two splits of the original dried samples were extracted and both identified with the laboratory number plus “m”.  The mineral sediments samples were separated from the basal hollow core segment and identified with the site number.  The feather moss samples were divided into two splits, identified with the site number plus “fm”.

Samples analysis –. The <0.063 mm fractions of the mineral sediment samples were separated by sieving, treated with aqua regia, and analyzed for selected elements using ICP-AES methods (Appendix E).  Peat samples were digested with nitric-perchloric acid digestion prior to ICP-AES analysis.  Peat samples were also subjected to loss-on-ignition analysis at 500 degrees C (Appendix D).  This analytical work on the peat and mineral sediment samples was undertaken at ALS Chemex Ltd., Vancouver.

One split of the feather moss samples was analyzed for selected elements using INAA methods (Appendix E).. The other was ashed and the residue treated with an aqua regia digestion, and analyzed using ICP-AES techniques methods.  Analytical work on the feather moss samples was undertaken at Activation Laboratories, Ancaster, Ontario.

#### EXPLANATION OF THE OPEN FILE REPORT AND DIGITAL RELEASE

The Open File 3882 publication comprises a report of 37 pages (plus a pocket item) with discussions of compositional data for peat, feather moss, and sediment data.  Also included are 9 Appendices, consisting of  (i) information on peatland forms and vegetation  (Appendix A); (ii) peatland site descriptions (Appendix B); (iii) field descriptions of hummock and hollow sequences (Appendix C); (iv) peat sample descriptions, including data for loss-on-ignition and bulk density measurements (Appendix D); (iv) geochemical data for peat, feather moss, and mineral sediment samples (Appendix E); (v) summary statistical data (Appendix F); (vi) geochemical data for duplicate samples (Appendix G); (vii) plots showing geochemical profiles, with depth, of peat in hummocks and hollows in 24 peatlands (Appendix H);  and (vii) plots showing the distribution of selected elements in three peat horizons and feather moss in peatlands in dominant and non-dominant wind sectors.

The digital release that accompanies Open File 3882 consists of the following files:

site.txt; site.xls – peatland site locations and descriptions of setting and surface vegetation (see Appendix B below).

fielddes.txt; fielddes.xls – field descriptions of  hummock and hollow sequences (see Appendix C below).

sampdes.txt; sampdes.xls – descriptions of peat samples from hummock and hollow sequences, includes Munsell colours and data for bulk density and loss-on-ignition  measurements.

peat_icp.txt; peat_icp.xls – peat geochemical data (ICP-AES) for samples collected from hummock and and hollow sequences in 24 peatlands ( see Appendix E below)

.

fm_icp.txt; fm_inaa.txt; fm_inaa.xls; fm_inaa.xls:  geochemical data (ICP-AES and INAA) for feather moss samples from 14 peatlands  (see Appendix E explanation).

sed_icp.txt; sed_icp.xls:  geochemical data (ICP-AES) for samples of mineral sediments underlying the hollow peat sequences (see Appendix E explanation).

|  | site.txt; site.xls | # ROUYN-NORANDA PEATLAND SITE DESCRIPTIONS |  |
| --- | --- | --- | --- |
|  | Site Number | Site Number (data generated for 24 sites numbered between |  |
|  |  | 97KFA3500 and 97KFA3540) |  |
|  | Sector | Position of the site with respect to the compass direction |  |
|  |  | from the smelter |  |
|  | Distance (km) | Distance from smelter in kilometres |  |
|  | Lat (deg) | Latitude (degrees) |  |
|  | Long (deg) | Longitude (degrees) |  |
|  | Location Description | Position of site with respect to the nearest town |  |
|  | Hummock Samples | Samples hand cut from hummock, with length in centimetres (yes or no) |  |
|  | Flat Peatland Samples | Surface and core samples from hollow or flat part of peatland, with length in centimetres (yes (with sampling depth) or no) |  |
|  | pH Water | pH of water at or near hummock base |  |
|  | Site Description | Description of peatland setting, vegetation, and underlying materials and date sampled. |  |

|  | fielddes.txt; fielddes.xls | # FIELD DESCRIPTIONS OF PEAT IN HUMMOCK AND HOLLOW SEQUENCES |
| --- | --- | --- |
|  | Site Number | Site Number (data generated for 24 sites numbered between |
|  |  | 97KFA3500 and 97KFA3540) |
|  | Location | Hummock - Sphagnum peat taking the form of a raised mound |
|  |  | Flat - peat in sequence from hollow (flat part of |
|  |  | peatland) adjacent to the hummock |
|  | Depth (cm) | Depth from the surface in centimetres |
|  | From | Depth at top of sample or core interval in centimetres |
|  | To | Depth at base of sample or core interval in centimetres |
|  | Sequence description | Description of peat materials with depth |

| sampdes.txt; sampdes.xls |  | PEAT SAMPLE DESCRIPTIONS |
| --- | --- | --- |
| Site Number/Sample Type |  | Site number with modifier "h" or "f"; "h" indicates sequence from hummock; "f" indicates sequence from flat part of peatland (hollow) adjacent to hummock. |
| Laboratory Number |  | Sample number assigned in the laboratory. Hummock sample numbers are designated with "w" or "s". Those samples with "w" are a split of the original sample and are composed of all hummock materials, except for coarse twigs and woody roots which were removed. Those samples with "s" were prepared from another split of the original hummock sample and are composed of hand-picked Sphagnum fragments only. Samples from the hollow sequence are designated with "m" and are composed of all peat materials, except for coarse twigs and woody roots. |
| Sector |  | Position of the site relative to the compass direction |
|  |  | from the smelter |
| Depth (cm) |  | Depth from the surface in centimetres |
| From |  | Depth at top of sample or core interval in centimetres |
| To |  | Depth at base of sample or core interval in centimetres |
| Moisture % (Wet Wt.) |  | Per cent water content, based on wet weight of sample |
| Moisture % (Dry Wt.) |  | Per cent water content, based on dry weight of sample |
| Bulk Den (gm/cm3) |  | Bulk density determination (gm/cm3) |
| Ash % |  | Ash remaining in sample after ignition at 500 degrees C. |
| Munsell Colour |  | Munsell colour of sample |
| Munsell or Other Colour Desc |  | Description of Munsell colour or observed colour |

| peat_icp.txt; peat_icp.xls fm_icp.txt; fm_icp.xls fm_inaa.txt; fm_inaa.xls sed_icp.txt; sed_icp.xls |  | GEOCHEMICAL DATA FOR PEAT, FEATHER MOSS AND SEDIMENT SAMPLES |
| --- | --- | --- |
| Site Number/Sample Type |  | Site number with modifier "h" or "f"; "h" indicates sequence from hummock; "f" indicates sequence from flat part of peatland (hollow) adjacent to hummock. |
| Laboratory Number |  | Sample number assigned in the laboratory. Hummock sample numbers are designated with "w" or "s". Those samples with "w" are a split of the original sample and are composed of all hummock materials, except for coarse twigs and woody roots that were removed. Those samples with "s" were prepared from another split of the original hummock sample and are composed of hand-picked Sphagnum fragments only. Samples from the hollow sequence are designated with "f" and are composed of all peat materials, except for coarse twigs and woody roots. |
| Sector |  | Position of the site relative to the compass direction |
|  |  | from the smelter |
| Dist (km) |  | Distance from the smelter in kilometres |
| Depth (cm) |  | Depth from the surface in centimetres |
| From |  | Depth at top of sample or core interval in centimetres |
| To |  | Depth at base of sample or core interval in centimetres |

| peat_icp.txt; peat_icp.xls fm_icp.txt; fm_icp.xls sed_icp.txt; sed_icp.xls |  |  | ICP-AES DATA |  |  |
| --- | --- | --- | --- | --- | --- |
| Element | Unit | Element Name |  | Lower Detection Limit | Upper Detection Limit |
| Ag | ppm | Silver |  | 0.10 | 100 |
| Al | ppm | Aluminium |  | 50 | 100000 |
| As | ppm | Arsenic |  | 2 | 10000 |
| Ba | ppm | Barium |  | 5 | 10000 |
| Ca | pct | Calcium |  | 0.005 | 10 |
| Cd | ppm | Cadmium |  | 0.2 | 100 |
| Co | ppm | Cobalt |  | 0.50 | 10000 |
| Cr | ppm | Chromium |  | 0.5 | 10000 |
| Cu | ppm | Copper |  | 0.50 | 10000 |
| Fe | pct | Iron |  | 0.005 | 10 |
| K | pct | Potassium |  | 0.005 | 1.00 |
| Mg | pct | Magnesium |  | 0.005 | 10 |
| Mn | ppm | Manganese |  | 1 | 10000 |
| Mo | ppm | Molybdenum |  | 0.50 | 10000 |
| Na | ppm | Sodium |  | 50 | 50000 |
| Ni | ppm | Nickel |  | 0.50 | 10000 |
| P | ppm | Phosphorus |  | 5 | 10000 |
| Pb | ppm | Lead |  | 0.50 | 10000 |
| Sb | ppm | Antimony |  | 2 | 10000 |
| Sr | ppm | Strontium |  | 0.50 | 10000 |
| Ti | ppm | Titanium |  | 0.005 | 10 |
| V | ppm | Vanadium |  | 0.5 | 10000 |
| Zn | ppm | Zinc |  | 0.50 | 10000 |

| fm_inaa.txt; fm_inaa.xls |  |  | INAA data |  |  |
| --- | --- | --- | --- | --- | --- |
| Element | Unit | Element Name |  | Lower Detection Limit |  |
| Ag | ppm | Silver |  | 0.2 |  |
| As | ppm | Arsenic |  | 0.5 |  |
| Au | ppb | Gold |  | 0.1 |  |
| Ba | ppm | Barium |  | 5 |  |
| Br | ppm | Bromine |  | 1 |  |
| Ca | pct | Calcium |  | 0.2 |  |
| Ce | ppm | Cerium |  | 3 |  |
| Co | ppm | Cobalt |  | 1 |  |
| Cr | ppm | Chromium |  | 1 |  |
| Cs | ppm | Cesium |  | 0.5 |  |
| Fe | pct | Iron |  | 0.05 |  |
| Hf | ppm | Hafnium |  | 0.5 |  |
| Hg | ppm | Mercury |  | 1 |  |
| K | pct | Potassium |  | 0.05 |  |
| La | ppm | Lanthium |  | 0.1 |  |
| Lu | ppm | Lutelium |  | 0.05 |  |
| Mo | ppm | Molybdenum |  | 0.005 |  |
| Na | ppm | Sodium |  | 10 |  |
| Sb | ppm | Antimony |  | 0.1 |  |
| Sc | ppm | Scandium |  | 0.1 |  |
| Sm | ppm | Samarium |  | 0.1 |  |
| Sr | ppm | Strontium |  | 10 |  |
| Zn | ppm | Zinc |  | 50 |  |