A REPORT ON THE 2001 TILL SAMPLING ON THE

KIKERK LAKE PROPERTY, NUNAVUT

on the

KL8, KL10, KL11 and KL12 Claims

F33508, F33510, F33511 and F33512.

(NTS mapsheets: 86I/14, 86P/03)

67 00’N, 113 00’W

WEST KITIKMEOT REGION

NUNAVUT

Prepared for:

DE BEERS CANADA EXPLORATION INC.

Prepared by:

Peter K. Holmes

Senior Geologist

May 2002

#### DECLARATION

I, Peter Holmes, certify that I have completed an Honours Bachelor of Science degree (Geology) in 1987 from Lakehead University, Thunder Bay, Ontario.

I have been involved in Geological Exploration since 1984 with Westfield Minerals Limited, Newmont Exploration Canada Limited and Monopros Limited.

Since 1989, I have been employed by:

De Beers Canada Exploration Inc.

Suite 300 – 5102 50th Avenue

Yellowknife, Northwest Territories

X1A 3S8

Peter Holmes

May 6th, 2002

# TABLE OF CONTENTS

### Page

- INTRODUCTION								5

- LOCATION AND ACCESS						5

- PROPERTY SUMMARY							7

- BEDROCK GEOLOGY							9

- SURFICIAL GEOLOGY							11

- 2001 EXPLORATION PROGRAMME

  - Till sampling							13

  - Sample Treatment						13

- INTERPRETATION OF 2001 SAMPLE RESULTS		16

- EXPLORATION EXPENDITURES					18

- CONCLUSIONS AND RECOMMENDATIONS			19

REFERENCES								20

# LIST OF FIGURES

## Figure 1		Location of Kikerk Lake Claims

Figure 2		Kikerk Lake Property

Figure 3		Regional Bedrock Geology of the Kikerk Lake Area

Figure 4		Location Map of 2001 Till Samples

Figure 5		Sample Results for 2001

# LIST OF TABLES

Table 1		Claims List

Table 2		Sample Treatment Weights

Table 3		Visual Sample Results

Table 4		2001 Kikerk Lake Project Expenditures

# LIST OF APPENDICES

Appendix 1		Till Sample Descriptions

Appendix 2		Project Expenditures and Personnel List

- INTRODUCTION

This report is a summary of the exploration activities in 2001 on the Kikerk Lake property (Figure 1) for De Beers Canada Exploration Inc (DBCE). Since the acquisition of the property in 1993, DBCE has actively explored the Kikerk property through sampling and geophysical surveys (Paul 1998; 1997; Paul and Ellemers 1995). Several kimberlite occurrences have been reported in Kikerk Lake area including the recent discovery of the Knife Lake pipe on the adjacent Tree-1 claim.

Field activities were carried out on July 21st, 2001, and only included till sediment sampling, and a detailed description of the surficial sediments.

All exploration activities were supported from the DBCE Rockinghorse Lake camp, some 130 kilometres south-southeast of the property. Field crews were transported to various parts of the property by Bell Jet Ranger helicopter.

- LOCATION AND ACCESS

The Kikerk Lake property is located south of the Coronation Gulf in the West Kitikmeot region, Nunavut some 530 kilometres north of Yellowknife and 95 kilometres to the southeast of Kugluktuk (previously Coppermine). The property currently comprises a contiguous block of four mineral claims and is centred on 67 00’ north latitude and 113 00’ west longitude to the south of Kikerk Lake.

Accommodation and support for field crews was available at the Rockinghorse Lake camp for the duration of the sampling programme and the surficial mapping exercise. Access to the Kikerk Lake property is possible only by fixed-wing aircraft or helicopter. No fuel is cached in the Kikerk Lake area so prospecting activities were restricted to a single visit to the property.

Once the field crew arrived on the first claim, KL8 the sampling was carried out by foot traverse. The helicopter was then used to transport the geologists to the next sample area on claim KL10. Due to weight restrictions on the helicopter only 10 samples could be collected.

- PROPERTY SUMMARY

The Kikerk Lake property (Figure 2) currently consists of four mineral claims (8 738.26 acres). All four mineral claims are in the process of being converted to exploration leases for eventual amalgamation with the Tree-1 claim (currently 100% Rhonda Corp). All claims or leases (Table 1) that comprise the Kikerk Lake property are registered to De Beers Canada Exploration Inc.

Table 1 – Claims List

| Claim No | Name | Area(ac) | Due Date | Work Req’d | Credit | NTS Map |
| --- | --- | --- | --- | --- | --- | --- |
| F33508 | KL8 | 2582.50 | 10-Feb-02 | $5165.00 | $307.56 | 86I/14 |
| F33510 | KL10 | 2582.50 | 10-Feb-02 | $5165.00 | $3906.96 | 86I/14 |
| F33511 | KL11 | 1033.00 | !0-Feb-03 | $0.00 | $9038.01 | 86I/14-86P/03 |
| F33512 | KL12 | 2540.26 | 10-Feb-03 | $0.00 | $4122.32 | 86I/14-86P/03 |

All four claims will reach their 10-year anniversary on February 10, 2003.

The Kikerk Lake claim block was first recorded by Mr Mike Magrum on February 10th, 1993.

These claims were subsequently transferred to Monopros Limited on February 10th, 1993.

The original Kikerk Lake block included 111 mineral claims. Through time 107 claims have been allowed to lapse bringing the total to its current total of four.

- BEDROCK GEOLOGY

The Kikerk Lake claim block lies within the Wopmay Orogen on the western margin of the Slave Structural Province of the northwestern Canadian Shield (Figure 3). The Wopmay Orogen (ca. 1.9Ga) is comprised of three tectonostratigraphic sequences. From west to east they are the Hottah Arc, Great Bear Arc and the Coronation Supergroup. The Wopmay Fault separates the Coronation Supergroup from the Great Bear Arc, and is marks the western limit of exposed Archean crust in the Canadian Shield. A brief summary from Hoffman (1989) and St-Onge (1991) describes the formation of the Wopmay Orogen. Shelf-rise and foredeep sediments of the Coronation Supergroup were deposited across the rifted western margin of the Slave Province. These sediments were transported eastward during convergence of the Slave Province and the Hottah Arc during the Calderian Orogeny. A subduction zone was established during the Slave-Hottah Arc collision that generated the intrusion of the Great Bear volcano-plutonic Arc. The Great Bear Arc unconformably overlies the Hottah Arc and the western edge of the Coronation Supergroup, along the Wopmay Fault zone. Finally the entire region was transected by a conjugate set of transcurrent faults.

Three distinctive stratigraphic sequences constitute the Coronation Supergroup. A rift-facies assemblage composed of both Akaitcho and Grant Groups is the first sequence. The second is a shelf-rise sequence called the Epworth Group and the third is the Recluse Group, a synorogenic foredeep sequence (St-Onge et al. 1991). Within the Kikerk Lake claim block there is no evidence of either the Akaitcho or Grant group. Only sediments of the Recluse and Epworth Groups of the Coronation Supergroup were observed within the Kikerk Lake area.

The eastern Epworth Group is composed of coarse shelf-facies, silici-clastics of the Odjick Formation such as quartzites and argillites. It also includes sediments of the Rocknest Formation that are generally cyclic dolomitic and argillitic units. Stromatolitic dolomites of the Rocknest Formation are widespread and are the most common lithology within the property.

The Recluse Group which overlies the Rocknest formation is classified as a flysch deposit. These deposits are generally thick sequences of deep marine sediments that were deposited during orogenic episodes (Prothero 1990). This group is comprised of four units including a thin layer of interbedded quartzite and argillite followed by thicker successions of black laminated argillite, turbiditic concretionary siltstones and laminated grey-green siltstones. Within the Kikerk Lake property, these units are present as north-south trending bands up to five

kilometres wide. The western and southern margins of Kikerk Lake are surrounded by Recluse Group sedimentary rocks. Rocks of the Archean Yellowknife Supergroup including autochthonous basement granites, gneisses and volcanic rocks are exposed in the extreme southeast corner of the property.

Dated at 1267 Ma (St-Onge et al 1991), the post-orogenic MacKenzie Dyke Swarm which is composed of the NNW-trending diabase dykes intrudes the Coronation Supergroup units and the underlying Archean basement (St-Onge et al 1991).

Structural features of significance within and surrounding the Kikerk Lake claim block include two fault sets associated with two of the five discrete deformational events that occurred in the region. The first set of north-striking thrust faults resulted from the first deformational event. One of these faults occurs just to the west of the Kikerk Lake claim block. The second set of faults are northeast-trending transcurrent or strike-slip faults. The post-orogenic faulting involved east-west shortening and north-south extension associated with the fifth (and final) deformational event (St-Onge et al 1991). These transcurrent faults offset the older thrust faults throughout the region.

- SURFICIAL GEOLOGY

The surficial geology of the Wopmay Orogen is a byproduct of the Keewatin sector of the Laurentide Ice Sheet. The Keewatin Ice Divide occupied a roughly north-south oriented region extending from the Manitoba-Nunavut border to Wager Bay (ca. 700 km). Ice originating from the Keewatin Ice Divide flowed from the east/northeast across the Slave Province and the Wopmay Orogen (Shilts and Aylsworth 1989). The Kikerk Lake property falls within the outermost of four zones distinguished by distinctive landforms and sediment assemblages that radiate outward from the ice divide. Zone four is characterized by extensive areas of nearly drift-free, ice-molded bedrock with virtually no esker development (Shilts and Aylsworth 1989).

A till veneer is the most extensive glacial deposit and only one stratigraphic unit has been recognized in the Kikerk Lake area. This till unit is typically thin, measuring less than two metres in thickness. It is generally presumed that these thin deposits underwent short transport distances and therefore very local sources are suggested. Bedrock exposures are common through the till cover. The matrix of the till tends to be very silty or sandy with a minor clay component. Consistent with the interpretation of short transport distances, the till tends to be clast-rich with angular cobbles and boulders. Few mudboils (the preferred sample medium) are observed in the Kikerk Lake area.

There are some areas of till blanket present in Kikerk Lake area. Till blanket is generally a lodgement or basal meltout till. It typically has a gentle rolling surface with thickness ranging up to 15 metres. The deposits are sometimes fluted and underlying bedrock topography is concealed by the thick till deposits (St-Onge 1988).

Glaciofluvial deposits although rare in the Kikerk Lake area do exist as eskers, kames and outwash plains. The sediment-starved eskers form small, sinuous ridges, parallel to sub-parallel with the ice transport direction. The esker systems follow a northwestern ice flow direction throughout most of the northwestern Slave Province and Wopmay Orogen. The ice flow direction becomes more northerly as the coast of the Coronation Gulf is approached. The ice direction is clearly reflected in the shape and orientation of the lakes and rivers.

- 2001 EXPLORATION PROGRAMME

6.1	Till Sample Collection

After a review of the previous sample results, a decision was made to collect some check samples within the Kikerk Lake property (Figure 4). A total of 10 till sediment samples were collected on July 21st, 2001.

Site selection concentrated on till material within this area of till veneer. All samples were field-screened to –20mm and a constant volume of 10 litres was collected. Detailed sample descriptions were recorded and the location was determined using a GPS receiver (Appendix 1). For this follow-up phase of sampling, material was collected at 100 metre intervals along lines that were approximately perpendicular to the dominant ice flow direction. A two-person crew (K Pollock and S Wollner) working in a Bell 206 helicopter completed the follow-up sampling.

All samples were flown to the De Beers Canada Exploration (DBCE) warehouse in Yellowknife for palletizing and onward transport to the DBCE processing facility in Sudbury, Ontario.

6.2	Sample Treatment

At the De Beers Canada treatment facility in Sudbury, the samples are first disaggregated and deslimed in modified cement mixers to remove the clay size fraction. The washed material is then screened to remove the oversize fraction (ie +1.0mm) and the ultrafine or undersized fraction (ie –0.3mm). The –1.0+0.3mm size fraction is treated through a mini-dense media separation (DMS) plant to produce an initial heavy mineral concentrate. Then the heavy mineral concentrate is passed through a micro-DMS plant (De Beers’ proprietary technology) to provide a sort-ready product. The weights of the various fractions are recorded for the different stages of sample processing (Table 2). This information can be used to define the textural characteristics of till in the Kikerk Lake area. On average, less than 1% by weight of the original sample is analyzed for kimberlitic indicator minerals.

Finally the dried heavy mineral concentrates are sent to the DBCE mineralogical unit in Toronto for kimberlitic mineral examination and classification.

Table 2 – Sample Treatment Weights

| Sample No | Initial Wt (kg) | Oversize Wt (kg) | Undersize Wt (kg) | Analytical Wt (kg) | Mini-DMS Conc Wt (g) | Micro-DMS Conc Wt (g) |
| --- | --- | --- | --- | --- | --- | --- |
| Y3350-01 | 13.70 | 4.14 | 0.59 | 2.19 | 434.4 | 80.8 |
| Y3351-01 | 13.00 | 3.12 | 0.48 | 2.22 | 292.8 | 49.3 |
| Y3352-01 | 12.89 | 3.99 | 0.44 | 1.85 | 242.6 | 46.8 |
| Y3353-01 | 13.34 | 4.84 | 0.52 | 1.96 | 354.0 | 56.9 |
| Y3354-01 | 12.15 | 4.00 | 0.46 | 1.90 | 294.1 | 75.6 |
| Y3355-01 | 12.77 | 3.80 | 0.61 | 1.96 | 313.9 | 53.4 |
| Y3356-01 | 13.20 | 3.70 | 1.02 | 2.14 | 549.6 | 139.8 |
| Y3357-01 | 13.01 | 2.32 | 0.47 | 2.08 | 159.7 | 28.2 |
| Y3358-01 | 14.43 | 3.78 | 0.48 | 2.18 | 256.0 | 54.2 |
| Y3359-01 | 12.51 | 4.34 | 0.75 | 2.56 | 488.1 | 115.0 |

- INTERPRETATION OF 2001 SAMPLE RESULTS

A total of four kimberlitic indicators were reported in the ten samples that were collected in 2001 (Table 3). All positive grains were found in the fine fraction (ie –0.5+0.3mm). Low grain counts coupled with the absence of coarse indicators is typically indicative of long distance transport in the glacial environment.

However two adjacent samples, Y3358-01 and Y3359-01 were both reported with single ilmenites with perovskite mantles (PM). These delicate PM textures are usually suggestive of proximity to their source.

A single spinel (chromite?) was noted in sample, Y3359-01 that may be derived from a kimberlite.

Although the indicator counts are low, the surface textures of the ilmenites suggest that the positive grains were derived from a proximal source.

Table 3 – Visual Sample Results

|  |  |  | Garnet |  |  | Ilmenite |  |  | Clinopyroxene |  | Spinel |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample Number | Size (mm) | Total | TGA | ROK | OTH | TIL | PM | OTH | TCD | ROS | TSP | OTH |
| Y3350 | -0.5+0.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3350 | -1.0+0.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3351 | -0.5+0.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3351 | -1.0+0.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3352 | -0.5+0.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3352 | -1.0+0.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3353 | -0.5+0.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3353 | -1.0+0.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3354 | -0.5+0.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3354 | -1.0+0.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3355 | -0.5+0.3 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3355 | -1.0+0.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3356 | -0.5+0.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3356 | -1.0+0.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3357 | -0.5+0.3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 |
| Y3357 | -1.0+0.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3358 | -0.5+0.3 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| Y3358 | -1.0+0.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Y3359 | -0.5+0.3 | 2 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 2 |
| Y3359 | -1.0+0.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 |

- EXPLORATION EXPENDITURES

Follow-up samples were collected within the Kikerk Lake property in 2001 as a check for kimberlitic indicator dispersion. The total expenditures (Appendix 2) of $12,646.75 for 2001 are claimed for representation work on claims KL8 and KL10 (to be grouped). A summary table of expenditures below (Table 4) lists the main cost centres and their allocations.

Table 4 – Kikerk Lake Project Expenditures in 2001

| Description of Expenditure | Amount (CDN$) |
| --- | --- |
| Staff Wages | 2110.00 |
| Accommodation and Food | 1015.00 |
| Helicopter costs | 1992.00 |
| Fixed Wing costs | 4530.00 |
| Sample Treatment at DBCE (Sudbury) | 700.00 |
| Transportation charges | 426.42 |
| Indicator Mineral Analyses | 829.10 |
| Administration overheads (9%) | 1044.23 |
| GRAND TOTAL | $12646.75 |

- CONCLUSIONS AND RECOMMENDATIONS

All four mineral claims will be converted to exploration leases due to their potential to host additional kimberlites and if any development associated with the Knife Lake pipe proceeds.

The results of the 2001 sampling suggest that undiscovered sources of indicator minerals may exist within the Kikerk Lake property. Follow-up sampling should be undertaken to better resolve the down-ice and up-ice dispersion of indicator minerals in the Kikerk Lake claims.

P.K. Holmes

May 6, 2002

REFERENCES

Hoffman, P.F., (1989) Precambrian geology and tectonic history of North America. In A.W. Bally and A.R. Palmer (eds.) The Geology of North America – An Oveview: Boulder Colorado, Geological Society of America, Volume A.

Paul, J., (1998) Sediment Sampling and Geophysics, Kikerk Lake Property, 1997. Monopros Limited in-house report, submitted as assessment report for representation work, 17 pages.

Paul, J., (1997) Sediment Sampling and Geophysics, Kikerk Lake Property 1996. Monopros Limited in-house report, submitted as assessment report for representation work, Mining Recorder Office report #83840.

Paul, J. and Ellemers, P. (1995) Kikerk Lake Claim Block, Sediment Sampling and Geophysics. Monopros Limited, In-house report submitted as assessment report for representation work, Mining Recorder Office report #83474.

Prothero, D.R. (1990) Interpreting the Stratigraphic Record. WH Freeman and Company, New York.

Shilts, W.W. and Aylsworth, J.M. (1989) Glacial Features around the Keewatin Ice Divide. Geololgical Survey of Canada, Paper 88-24.

St-Onge, D.A. (1988) Surficial Geology, Coppermine River, District of Mackenzie, Northwest Territories. Geological Survey of Canada, Map 1645A, 1:250 000.

St-Onge, M.R., King, J.E. and Lalonde, A.E. (1991) Geology,East-Central Wopmay Orogen, District of Mackenzie, Northwest Territories, Map 1754A, 1:250 000

APPENDICES

APPENDIX 1

Till Sample Descriptions

| Sample Number | Volume (liters) | Easting | Northing | Sample Depth | Colour | Landform | NTS Map | Material Type | Samplers | REMARKS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y3350 | 10 | 408194 | 7429897 | 0.3m | Grey | Till Veneer | 86I/14 | Till | KP/SW | Gentle topography and relief on boulder strewn plain. Clay-rich, crumbly and hard to sieve with a moderate amount of oversize material. |
| Y3351 | 10 | 408242 | 7429850 | 0.3m | Grey | Till Veneer | 86I/14 | Till | KP/SW | Gentle topography and relief on boulder strewn plain. Clay-rich and hard to sieve with a moderate amount of oversize material. |
| Y3352 | 10 | 408287 | 7429898 | 0.3m | Grey | Till Veneer | 86I/14 | Till | KP/SW | Gentle topography and relief on boulder strewn plain. Clay-rich, crumbly and hard to sieve with a moderate amount of oversize material. |
| Y3353 | 10 | 408336 | 7429940 | 0.3m | Grey | Till Veneer | 86I/14 | Till | KP/SW | Gentle topography and relief on boulder strewn plain. Clay-rich and hard to sieve with a moderate amount of oversize material. |
| Y3354 | 10 | 408381 | 7429973 | 0.3m | Grey | Till Veneer | 86I/14 | Till | KP/SW | Gentle topography and relief on boulder strewn plain. Clay-rich and hard to sieve with a moderate amount of oversize material. |
| Y3355 | 10 | 408351 | 7430018 | 0.3m | Grey | Till Veneer | 86I/14 | Till | KP/SW | Gentle topography and relief on top of a low rise in boulder strewn plain. Clay-rich, hard to sieve with moderate amount of oversize material. |
| Y3356 | 10 | 408299 | 7429971 | 0.3m | Grey | Till Veneer | 86I/14 | Till | KP/SW | Gentle topography and relief on boulder strewn plain. Clay-rich, compact, hard to sieve with little oversize material. |
| Y3357 | 10 | 408258 | 7429920 | 0.3m | Grey | Till Veneer | 86I/14 | Till | KP/SW | Low topography and relief in boulder strewn plain with blocky erratics. Clay-rich, compact, hard to sieve with little overscreen material. |
| Y3358 | 10 | 406513 | 7430706 | 0.3m | Grey | Till Veneer | 86I/14 | Till | KP/SW | Low topography and relief in boulder strewn plain. Clay-rich, compact, hard to sieve with abundant overscreen material. |
| Y3359 | 10 | 406454 | 7430657 | 0.3m | Grey | Till Veneer | 86I/14 | Till | KP/SW | Low elevation and relief in boulder strewn plain. Clay-rich with abundant oversize. |

NB All UTM coordinates are based on Zone 12, NAD27.

APPENDIX 2

PROJECT EXPENDITURES AND PERSONNEL LIST

###### KIKERK LAKE PROJECT - 2001

##### Sampling Project Expenditure Calculations

Total samples = 10

LIST OF PERSONNEL

The following personnel contributed to the work performed on the Kikerk Lake property:

Geologists:

Peter Holmes (Senior Geologist)	De Beers Canada Exploration Inc

Kari Pollock (Field Geologist)		300 – 5102 50th Avenue

Yellowknife, NT  X1A 3S8

Contractors:

John Avaligik (camp attendant)

Sage Rich (cook)

Sue Wollner (contract geologist)

Helicopter Pilot:

Diana Taylor

Helicopter Engineer:

Steve Camphaug

Addresses for all the students will be the De Beers Yellowknife address noted above. All these students were employed by DBCE and were working on the Kikerk/Rockinghorse Lake project from July 1 to 23.

Expediter:	G&G Expediting Limited

Sub PO #1, Yellowknife, NT  X1A 2S9

Fixed Wing:	Air Tindi Ltd.

Box 1693, Yellowknife, NT  X1A 2P3

Helicopter:	Great Slave Helicopters

Bag 7500, Yellowknife, NT  X1A 2R3